CRYSTALS-Dilithium NTT Engine using PYNQ Z2

Project Overview

This project set out to build a hardware accelerator for the Number Theoretic Transform used inside CRYSTALS-Dilithium, targeting a real deployment on the PYNQ-Z2 platform. The goal was not just to simulate cryptographic math but to convert a research algorithm into a working FPGA IP core that can be called from software like a real cryptographic co-processor. At this stage the core objective has been achieved: a complete NTT accelerator has been designed in Vitis HLS, verified through C simulation and RTL co-simulation, packaged as a Vivado IP, integrated with the Zynq processing system, and accessed from Python on the board.

Project Workflow

The project started with a clean HLS implementation of a 256-point modular NTT using the Dilithium prime (8380417). The hardware design employs AXI master ports to fetch inputs and twiddle factors into on-chip BRAM, processes them via a pipelined butterfly network, and writes results to DDR. Extensive work resolved interface, depth, and testbench issues, leading to successful C simulation, C synthesis, and RTL co-simulation. The co-simulation pass was a key achievement, validating that the generated RTL accurately matches the reference software model.

The Zynq-7020 NTT accelerator uses minimal resources (4 DSPs, 4 BRAMs, ~15k FF, ~13k LUTs) and runs at 100 MHz. Its latency is ~109k cycles (1ms per NTT), offering orders of magnitude acceleration over a Python ARM implementation and freeing the main processor. The AXI interface supports efficient burst transfers for polynomial vectors.

The project successfully delivered a functional FPGA NTT engine, including a verified HLS design, performance metrics, a reusable Vivado IP core, and working PYNQ software. The next phase will focus on system-level validation: comparing with software libraries, adding inverse NTT, integrating into a complete Dilithium pipeline (key generation/signing), and benchmarking power and throughput against ARM execution.

### Design Quality Analysis

Timing closure shows minor negative slack which can be resolved with floorplanning or clock reduction to 95 MHz. AXI transactions are inferred as full bursts, minimizing PS-PL traffic overhead.

The current architecture favors clarity and correctness over extreme parallelism, which is appropriate for a first silicon-proven design. The modular structure allows later extension to inverse NTT and Montgomery domain operations.

### Deliverables

### Stable Algorithm Implementation

- A reference software NTT was created and verified against random vectors.

- Modular arithmetic for q = 8380417 was implemented using constant-time reduction.

HLS Hardware Design

- Top function exposes AXI-MM ports for input vector, output vector, and twiddle table.

- Internal BRAM buffers added for double-buffered streaming.

- Pipelined butterfly stage with II=1 achieved.

- Depth pragmas inserted to satisfy co-simulation requirements.

Board-Level Validation

- Overlay loaded successfully on PYNQ-Z2.

- Control registers respond correctly.

- Accelerator execution confirmed through AXI-Lite polling.

- End-to-end transform matches software reference.

### Performance Summary

Transform size: 256 coefficients

Latency: 109,282 clock cycles

Clock: 100 MHz

Time per NTT: ≈ 1.09 ms

Interface: AXI memory + AXI-Lite control

Memory Architecture

- Two 256×32 BRAM banks for data and twiddles

- AXI burst length 256 for input and output

- Single AXI master shared across ports

- Pipelined copy in and copy out engines

### Next Steps

- Add inverse NTT to support full polynomial multiplication

- Introduce streaming AXI-DMA version for higher throughput

- Quantify energy per transform

- Integrate with Dilithium signing pipeline

- Explore 4-lane parallel butterfly for sub-0.3 ms latency